# Monte Carlo Radiative Exchange Simulator

A web-based tool for modeling radiative heat transfer between two plates when one surface is micro-structured. The simulator combines 3-D Monte Carlo ray tracing, temperature-dependent thin-film optics, surface-roughness scattering, waveguide modal physics, offline rigorous diffraction (RCWA), Polder–Van Hove near-field transfer, and an active structured near-field Green-tensor + LDOS correction (Track B2) to quantify anisotropic energy decoupling — how micro-structured surfaces can absorb incoming radiation efficiently while emitting thermal radiation poorly from inside their cavities.

## 🎯 The Core Idea

Most surfaces absorb and emit radiation equally (Kirchhoff's law). But micro-structured surfaces break this symmetry:

- Top surface: Incoming light → absorbed efficiently (α_eff ≈ 0.9–0.99)

- Inside cavities: Thermal photons → mostly trapped and recycled (ε_B ≈ 0.01–0.1)

- Result: Decoupling ratio up to 100:1, enabling advanced thermal engineering

This simulator runs photon-scale Monte Carlo experiments inside repeating cavity cells, accounting for:

- Thin-film effects — 100 nm walls emit 35× less than bulk

- Waveguide cutoff — photons longer than cavity diameter decay inside

- Complex optics — wavelength-dependent refractive indices and interference

- Temperature-dependent optics — Drude–Lorentz n(λ,T) with phonon/electron damping that grows with temperature, plus non-local corrections for nanostructures

- Surface-roughness scattering — Beckmann–Spizzichino / Harvey–Shack BRDF replaces naive Lambertian reflection when roughness is supplied

- Near-field transfer — evanescent waves tunnel across tiny gaps; structured Green-tensor LDOS correction (Track B2) enhances sub-wavelength inter-plate channel

- Thermal radiosity — combined heat flow across the complete enclosure

## 🧪 Key Physics

| Feature | How it works | Why it matters |
| --- | --- | --- |
| Thin-film Beer–Lambert | Walls thinner than absorption depth: ε_eff(λ) = ε_bulk × [1 − exp(−t/δ)] | 100 nm alumina: 2.2% vs. 80% (36× more accurate) |
| Complex refractive index + TMM | Full wavelength-dependent n(λ) + ik(λ) with phase-coherent Fresnel reflections | Captures Fabry–Pérot interference, oblique angles, s/p polarization |
| Temperature-dependent optics | Drude–Lorentz dielectric model with γ(T) damping and Varshni band-gap shift; anchored on 300 K tables via relative-drift factors | Hot walls (600–1200 K) become more lossy/emissive — correct high-T behavior without new lookup tables |
| Non-local hydrodynamic correction | Adds −β²k² to the Drude denominator when features are smaller than the electron mean free path (10–40 nm) | CNT forests (≈10 nm) scatter electrons off surfaces; local-response optics underestimates their absorption change |
| Surface-roughness BRDF | Beckmann–Spizzichino specular fraction exp[−(4πσcosθ/λ)²] + Gaussian facet lobe + Harvey–Shack diffuse aureole | Real etched surfaces are not Lambertian; σ→0 gives mirrors, σ≫λ gives diffuse |
| Lossy waveguide modal cutoff | Photons with λ > λ_c become evanescent; decay as exp(−2L/δ_ev) | Quantifies modal confinement; TE11: λ_c = 1.706 × diameter |
| Spectral Planck sampling | Each photon gets wavelength from thermal distribution; per-photon corrections applied | Accurate temperature-dependent behavior; 5-band model with Planck weighting |
| Polder–Van Hove near-field | For gaps < λ_peak/(2π): evanescent tunneling adds 2–100× flux | Auto-detected; seamless switch between near/far-field regimes |
| Structured near-field Green-tensor + LDOS (Track B2) | Cavity Green-function LDOS weighting of evanescent transmission; multiplicative nf_correction ≥ 1 applied symmetrically to inter-plate one-way fluxes | Active only in sub-wavelength regime; defaults to 1.0 (no-op) far-field; backward compatible |
| Four-surface radiosity | Plate A front/back + Plate B + surroundings; exact 3-D view factors | Net heat flow including reflections and enclosure effects |
| Offline RCWA diffraction | Fourier-modal-method S-matrix for periodic pitch p ~ λ; per-order propagating/evanescent labels; emits cached R/T/A tables | Captures diffraction the geometric ray tracer cannot; runs offline so web requests stay fast |

### Geometry Modes

- Honeycomb Cavity Panel — cylindrical pores in hexagonal array (porous anodized alumina)

- CNT Forest Panel — tapered carbon nanotube pillars in square lattice

- Legacy: Frustum cavities and rectangular pits (backward compatible)

## 📊 Key Output Metrics

| Metric | Meaning | Interpretation |
| --- | --- | --- |
| α_eff | Effective absorptivity for incident light | Higher = better light trapping; typically 0.8–0.99 |
| ε_B | Effective emissivity from cavity | Lower = better thermal confinement; typically 0.01–0.1 |
| Decoupling ratio | α_eff / ε_B | >10 means strong anisotropy; this is the "magic" of the structure |
| p_esc | Probability photon escapes cavity | How many thermally emitted photons get out; typically 0.001–0.1 |
| Net flux (Q) | Heat flow from plate A to plate B | Watts/m²; sign tells you direction |
| T_B_stag | Adiabatic equilibrium temperature of plate B | Temperature plate B would reach if isolated; useful for validation |
| physics_regime | "Near-field" or "far-field" | Near-field only triggers when gap is extremely small |
| Cutoff wavelength (λ_c) | Longest wavelength that propagates | λ_c = 1.706 × cavity_diameter; longer wavelengths are evanescent |

## 🏗️ System Architecture

### Data Flow

### Component Hierarchy

### Single Simulation Run (Inside the Engine)

## 📁 Project Structure

MonteCarlo ray tracing/ │ ├── 📄 README.md                                    # You are here ├── 📄 simulation_architecture.md                   # Plain-English architecture guide ├── 📄 DEVELOPMENT_NOTES.md                         # Physics phases & validation history ├── 📄 peer_review.txt                              # Physics references & citations │ ├── 🖥️  FRONTEND │   ├── templates/index.html                        # Web form & results display │   ├── static/app.js                               # Browser API client │   └── static/style.css                            # UI styling │ ├── 🔗 API & ORCHESTRATION │   ├── app.py                                      # Flask server, /api/simulate endpoint │   └── simulator.py                                # Main orchestration + radiosity model │ ├── ⚙️  GEOMETRY │   └── geometry.py                                 # Honeycomb, CNT forest, frustum cells │ ├── 🎲 MONTE CARLO ENGINE │   ├── ray_tracer.py                               # 3-D photon tracing in cavities │   ├── sampling.py                                 # Lambertian direction & Planck wavelength sampling │   └── spectral.py                                 # 5-band spectral model, Planck weighting │ ├── ✨ PHYSICS CORRECTIONS │   ├── material_optics.py                          # Thin-film Beer–Lambert, TMM, complex n+ik, │   │                                               #   Drude–Lorentz n(λ,T), non-local correction │   ├── brdf.py                                     # Beckmann–Spizzichino / Harvey–Shack roughness BRDF │   ├── waveguide_modes.py                          # TE11 modal dispersion, evanescent decay │   └── near_field_radiative_heat.py                # Polder–Van Hove near-field transfer │ ├── 🌊 FULL-WAVE ADAPTER + OFFLINE RCWA (PHASE 6) │   └── wave_physics/ │       ├── __init__.py │       ├── cached_solver.py                        # CachedWaveSolver: interpolate α_eff/ε_B │       ├── schemas.py                              # WaveResponse data structure (R/T/A grids) │       ├── rcwa.py                                 # Offline 1-D RCWA solver → cache producer (CLI) │       ├── analytic_benchmarks.py                  # Multilayer Fresnel reference │       ├── conventions.py                          # Sign & time-convention constants │       └── cache/ │           ├── default_wave_response.json          # Demo wave response table │           └── rcwa_demo_response.json             # RCWA-generated diffraction table │ ├── 📚 LEGACY / REFERENCE (NOT ACTIVELY USED) │   ├── monte_carlo_radiation.py                    # 2-D reference implementation │   └── rad_leakage.py                              # 2-D educational model │ ├── 🧪 TESTS & VALIDATION │   ├── test.py                                     # Core regression tests │   ├── test_thin_film.py                           # Thin-film physics validation │   ├── test_material_temperature.py                # Phase 4a: n(λ,T) & non-local validation │   ├── test_brdf.py                                # Phase 4b: Beckmann/Harvey–Shack BRDF validation │   ├── test_rcwa.py                                # B1: RCWA conservation/Fresnel/cache tests │   ├── test_wave_benchmarks.py                     # Wave response schema & cache tests │   ├── test_peer_review_physics.py                 # Peer-review physics checks │   ├── test_phase_integration.py                   # Phase 1 & 2 integration tests │   ├── test_phase3_integration.py                  # Phase 3 (near-field) integration │   ├── test_phase3_api.py                          # Flask /api/simulate near-field tests │   └── validate_integration.py                     # Smoke tests across geometries │ ├── 📋 CONFIG │   ├── requirements.txt                            # Flask, NumPy, SciPy dependencies │   ├── .gitignore                                  # Git ignore patterns │   └── LICENSE                                     # MIT License

## 🚀 Quick Start

### 1. Set up environment

# Clone or download the repo, then navigate to itcd"MonteCarlo ray tracing"# Create a virtual environment (recommended) python -m venv .venv  # Activate it#   Windows PowerShell:  .\.venv\Scripts\Activate.ps1#   Windows cmd.exe:     .venv\Scripts\activate.bat#   macOS/Linux:         source .venv/bin/activate

### 2. Install dependencies

pip install -r requirements.txt # Requires: Flask >= 3.0, NumPy >= 1.26, SciPy >= 1.12

### 3. Run the app

python app.py # Opens at http://127.0.0.1:5000/ in your browser

### 4. Use the web interface

- Select geometry: Honeycomb Cavity Panel or CNT Forest Panel

- Set Plate A (source): temperature, emissivity, dimensions

- Set Gap & environment: distance between plates, surroundings temperature

- Set Plate B cavity: depth, material properties

- Set MC parameters: photon count (1000–200,000), solver mode (Ray or Cached)

- Click "Run Simulation"

- Review results: α_eff, ε_B, decoupling ratio, net flux, physics regime badge

## 📊 Using the Web Interface

The simulator UI is organized in collapsible sections:

### Plate A Configuration (Source / Upper Plate)

- Temperature (K): Default 600 K (hot source)

- Emissivity: Front surface (default 1.0); typical range 0.1–1.0

- Back emissivity: Rear surface (default 0.1); often lower to reduce losses

- Material: Optional spectral material (silver, gold, etc.)

- Dimensions: Width and depth in µm (default 1000 × 1000 µm²)

### Gap & Environment

- Gap distance (µm): Space between Plate A and Plate B cavity opening (default 100 µm)

  - Small gaps (< λ_peak/2π) trigger near-field physics automatically

  - Larger gaps use classical far-field radiosity

- Plate B temperature (K): Default 300 K (cooler)

- Surroundings temperature (K): Background radiation temperature (default 300 K)

### Plate B Cavity Configuration

Honeycomb mode:

- Cavity diameter (µm): Width of cylindrical pores (default 20 µm)

- Wall thickness (µm): Distance between pore centers (default 1 µm)

- Cavity height (µm): Depth of pores (default 450 µm)

- Wall emissivity: Side walls (default 0.95)

- Base emissivity: Bottom surface (default 0.95)

- Flat top emissivity: Between-pore surface (default 0.1)

CNT Forest mode:

- CNT pitch (µm): Lattice spacing (default 0.05 µm = 50 nm)

- Base diameter (nm): CNT width at bottom (default 10 nm)

- Top diameter (nm): CNT width at tip (default 5 nm, tapered)

- CNT height (µm): Forest depth (default 450 µm)

- Material emissivities: Similar to honeycomb

- Material: Optional spectral material

### Monte Carlo Settings

- Photon count: How many rays to launch (1,000–200,000; more = more accurate but slower)

- Solver mode:

  - Ray: Full Monte Carlo (default; accurate but slower ~10–30 s)

  - Cached: Use pre-computed wave response table (fast; ~1 s)

- Full-gap MC (optional): Run separate Monte Carlo view-factor verification

- Enable near-field: Auto-detect near-field regime (recommended: ON)

### Results Display

After simulation completes:

Anisotropy Metrics:

- α_eff: How well the cavity absorbs incoming light (0–1)

- ε_B: How well the cavity emits thermal radiation (0–1)

- Decoupling ratio: α_eff / ε_B (higher = stronger anisotropy)

Heat Transfer:

- Net flux A→B: Watts/m² flowing from plate A to B

- Stagnation temperature: Temperature plate B would reach if isolated

Diagnostics:

- Physics regime: "⚡ NEAR-FIELD MODE" or "📡 FAR-FIELD MODE"

- Cutoff wavelength: λ_c = 1.706 × cavity_diameter (photons longer than this are evanescent)

- Confinement %: How many thermal photons are trapped inside the cavity

- 95% confidence intervals: Statistical uncertainty on α_eff and ε_B

## � REST API

If you want to call the simulator programmatically:

### Endpoint

POST /api/simulate Content-Type: application/json

### Essential Request Fields

{"geometry_mode":"honeycomb",// or "cnt_forest""cavity_diameter":20.0,// µm (honeycomb)"wall_thickness":1.0,// µm (honeycomb)"height":450.0,// Cavity depth (µm)"temp_a":600.0,// Plate A temperature (K)"emissivity_a":1.0,// Plate A emissivity (0–1)"emissivity_a_back":0.1,// Plate A back emissivity"gap":100.0,// Gap distance (µm)"temp_b":300.0,// Plate B temperature (K)"temp_surr":300.0,// Surroundings temperature (K)"n_photons":5000,// Monte Carlo photon count"wave_model":"ray",// "ray" or "cached""enable_near_field":true,// Auto-detect near-field"surface_roughness_um":0.05,// OPTIONAL: RMS roughness σ (µm); omit → Lambertian"roughness_correlation_um":1.0// OPTIONAL: correlation length τ (µm)}

Note on temperatures: Plate A/B temperatures (temp_a, temp_b) automatically drive temperature-dependent optical constants — no extra flag needed. At 300 K the tabulated Palik data are used exactly; at other temperatures a Drude–Lorentz drift model applies.

### Example Response

{"status":"success","results":{"alpha_eff":0.9101,"alpha_eff_ci95":0.0045,"epsilon_b":0.0710,"epsilon_b_ci95":0.0105,"decoupling_ratio":12.8,"p_esc":0.0072,"net_flux_A_front":-6.6,"q_net_a_to_b_physical":-6.6,"T_B_stag":284.75,"cutoff_wavelength_um":853.0,"confinement_pct":98.7,"cavity_enhancement":161.0,"physics_regime":"far-field","gap_ratio":130.1,"wave_model":"ray","solver_mode":"ray","kirchhoff_error":0.0}}

See the code in app.py for the complete list of optional parameters (material selections, near-field quadrature settings, etc.).

## 🧬 Key Physics Equations

### Thin-Film Beer–Lambert Law

A wall thinner than the absorption depth emits far less than bulk:

ε_eff(λ) = ε_bulk × [1 − exp(−t/δ(λ))]

Example: 100 nm alumina at λ = 10 µm (absorption depth δ ≈ 3.61 µm):

ε_eff = 0.80 × [1 − exp(−0.1/3.61)] ≈ 0.022 (2.2%)

This is a 36× correction from the naive bulk value (0.80).

### Waveguide Cutoff Wavelength

Photons longer than cutoff cannot propagate; they become evanescent (trapped):

λ_c = 1.706 × d_cavity    (circular cavities, TE11 mode) λ_c = 2 × min(w, d)        (rectangular cavities, TE10 mode)

Evanescent photons decay as:

T = exp(−2L/δ_ev),  where  δ_ev = (λ_c/2π) / √(1 − (λ_c/λ)²)

### Spectral Emissivity (Planck-Weighted)

The effective emissivity at a given temperature integrates across all wavelengths:

ε_eff(T) = ∫ ε(λ) × M_λ(λ,T) dλ / σT⁴

where M_λ is the Planck spectral radiance.

### Near-Field Regime Detection

For very small gaps, evanescent waves tunnel across the gap. The gap ratio tells you:

gap_ratio = g / (λ_peak / 2π),  where  λ_peak = 2898 µm·K / T

- gap_ratio < 1: Strong near-field (10–100× classical flux)

- gap_ratio 1–5: Moderate near-field (2–10× flux)

- gap_ratio 5–20: Weak near-field (1–2× flux)

- gap_ratio > 20: Far-field (classical limit)

Default threshold is 5.0; if gap_ratio < 5, the Polder–Van Hove model is used.

When the near-field regime is active, an additional structured near-field Green-tensor + LDOS correction (Track B2) multiplies the inter-plate channel by a symmetric enhancement factor nf_correction ≥ 1.0 (cached in a NearFieldResponse table over wavelength × gap). For bundled materials with no strong surface-polariton resonance (e.g. Al₂O₃ at 300–600 K) this factor is clamped to 1.0; for SPhP-supporting media (SiC, SiO₂) it yields 1.5–5× enhancement. Far-field defaults are unchanged.

### Temperature-Dependent Optical Constants (Drude–Lorentz)

The 300 K tables remain authoritative; a parametric model supplies the relative drift:

ñ(λ,T) = ñ_tab(λ) · f,   with per-component factors f_n = Re√ε_model(λ,T) / Re√ε_model(λ,300K),   f_k = Im√ε_model(λ,T) / Im√ε_model(λ,300K)

The dielectric model by material class:

Drude metal / polar phonon:  ε(ω,k,T) = ε∞ − ωp² / (ω² + iωγ(T) − β²k²) Semiconductor (Varshni):     E_g(T) = E_g(0) − αT²/(T + β) γ(T) = γ₃₀₀·[1 + s·(T/300 − 1)]                  (electron–phonon damping)

Non-local correction — when a structure's characteristic size L is below the electron mean free path ℓ (e.g., 10 nm CNTs vs. ℓ ≈ 25 nm), the hydrodynamic term activates with confinement wavevector k = π/L plus Fuchs–Sondheimer surface scattering:

γ_eff = γ(T)·[1 + 3(1−p)ℓ/(8L)],   β = √(3/5)·v_F

At T = 300 K with no sub-MFP feature the factors are exactly 1 → tabulated data returned.

Example: silver at λ = 10 µm, T = 900 K → collision rate grows ~2.3×, n rises toward 1 (Hagen–Rubens reflectivity drop); alumina phonon damping doubles k, halving the absorption depth.

### Surface-Roughness BRDF (Beckmann–Spizzichino / Harvey–Shack)

Each bounce splits reflected power between three lobes:

p_spec = exp[−(4πσcosθᵢ/λ)²]        → exact mirror direction TIS    = 1 − exp[−(4πσ/λ)²]         → incoherent total integrated scatter lobe   : Gaussian facets, rms slope m = √2·σ/τ, around the mirror direction rest   : Lambertian diffuse (Harvey–Shack aureole limit)

- σ → 0⁺: perfect mirror; σ ≫ λ: fully diffuse; energy conservation holds (the split is directional only — photon weights are untouched).

- Omitting surface_roughness_um restores the legacy pure-Lambertian bounce exactly.

### RCWA Diffraction (offline)

For periodic pitch p comparable to λ, the app ships an offline Fourier-modal-method solver:

E(x,z) = Σ_i S_i(z)·exp(−i·k̄x_i·x),   k̄x_i = n₁sinθ + i·λ/p TE: S'' = (D² − [ε])S      TM: S'' = (D·G·D·E − E)S,  G = [1/ε]^∧

It emits per-order diffraction efficiencies with propagating/evanescent labels into a WaveResponse cache consumed by the existing cached solver mode — no heavy electromagnetics ever run inside the Flask request thread. Regenerate the bundled demo table:

python -m wave_physics.rcwa --pitch-um 20 --height-um 100 --fill 0.85 --overwrite

## ✅ Validation Checklist

The simulator includes built-in energy conservation and physics validation. All output JSON includes a kirchhoff_error field (should be near 0). To manually verify results:

### Sanity Tests

| Test | How to check | Expected result |
| --- | --- | --- |
| Flat-plate limit | Set cavity_height → 0 | Compare to parallel-plate Stefan–Boltzmann formula |
| Blackbody limit | Set all ε = 1.0 | α_eff = ε_B = 1.0; decoupling = 1 |
| Thermal equilibrium | Set T_A = T_B = T_C | net_flux → 0 (within ~0.1%) |
| Solve swap | Run with wave_model='ray' then 'cached' | Results should match (within ~1% error) |
| Cutoff verification | Check cutoff_wavelength_um | Should equal 1.706 × cavity_diameter |
| Energy conservation | Check kirchhoff_error | Should be ≤ 0.1% |

### Run the Test Suite

# Core regression tests python test.py  # Thin-film physics python test_thin_film.py  # Temperature-dependent & non-local optics (Phase 4a) python test_material_temperature.py  # Surface-roughness BRDF (Phase 4b) python test_brdf.py  # RCWA diffraction solver (B1) python test_rcwa.py  # Full-wave caching python test_wave_benchmarks.py  # Physics checks python test_peer_review_physics.py  # Integration tests python test_phase_integration.py python test_phase3_integration.py python test_phase3_api.py  # Cross-geometry smoke tests python validate_integration.py

## � Accuracy & Limitations

### Current Accuracy

| Metric | Accuracy | Note |
| --- | --- | --- |
| Wall ε_eff | < 5% vs. literature | 100 nm alumina: 2.2% validated |
| Cutoff wavelength | < 1% vs. theory | TE11 mode formula |
| Statistical CI | ±5–10% relative | 95% confidence intervals reported |
| Energy conservation | < 0.1% | Kirchhoff reciprocity verified |
| Near-field vs. far-field | Tested & validated | Auto-detection threshold = 5.0 |

### Strengths ✓

- Thin-film Beer–Lambert + TMM optics (4-material database)

- Temperature-dependent Drude–Lorentz optics with non-local hydrodynamic correction

- Surface-roughness BRDF (Beckmann–Spizzichino / Harvey–Shack) — optional, energy-conserving

- Lossy waveguide modal analysis with real complex propagation constants

- Spectral integration with wavelength-dependent absorption

- Polder–Van Hove near-field with automatic regime detection

- Structured near-field Green-tensor + LDOS correction (Track B2) — active nf_greens solver

- Offline RCWA diffraction solver with validated per-order energy conservation

- Cached full-wave service layer with energy-conserving response tables

- Multi-scale physics: cavity behavior → enclosure heat transfer

- 95% confidence intervals on all Monte Carlo outputs

### Known Limitations ✗

- Geometric-optics base: Full electromagnetic solutions are accessed via the cached solver mode (RCWA runs offline; not in-thread)

- 1-D RCWA approximation: The periodic diffraction solver models the 2-D pore lattice via an effective fill fraction (provenance-labeled in every cache); full 2-D unit-cell RCWA is future work

- Roughness opt-in: Without surface_roughness_um, reflection stays purely Lambertian (legacy behavior)

- Periodic idealization: Ignores edge effects and non-uniformities at sample boundaries

- Material data: From literature (Palik handbook, etc.); temperature drift above 300 K is model-based (Drude–Lorentz/Varshni anchoring), not sample-specific measurements

- Solver timeout: Near-field integration (PVH + Green-tensor LDOS) falls back to far-field gracefully on numerical issues; the correction factor defaults to 1.0 if the cache builder fails

## �📚 References

### Thin-film / optical

- Born & Wolf, Principles of Optics (1999); Heavens (1955); Palik, Handbook of Optical Constants of Solids (1998).

### Temperature-dependent & non-local optics

- Modest, Radiative Heat Transfer, 3rd ed. (2013) — high-temperature dielectrics.

- Raza et al., J. Phys.: Condens. Matter 23, 325502 (2011) — hydrodynamic non-local response; Varshni, Physica 34, 149 (1967) — band-gap shift.

### Surface roughness / BRDF

- Beckmann & Spizzichino, The Scattering of Electromagnetic Waves from Rough Surfaces (1987); Harvey (1976); Stover, Optical Scattering (2012); Siegel & Howell, Thermal Radiation Heat Transfer — mixed specular/diffuse enclosure models.

### Rigorous diffraction (RCWA)

- Moharam & Gaylord, JOSA 71, 811 (1981); Moharam et al., JOSA A 12, 1077 (1995) — stable formulation; Lalanne & Morris, JOSA A 13, 779 (1996) and Li, JOSA A 13, 1870 (1996) — Fourier factorization rules; Whittaker & Culshaw, Phys. Rev. B 60, 2610 (1999) — scattering-matrix treatment.

### Anisotropic decoupling

- Lin et al., Phys. Rev. B 62, 3081 (2000); Narayanaswamy & Chen, Phys. Rev. B 70, 125101 (2004); Sprafke et al., Adv. Opt. Mater. 1, 527 (2013); Mizuno et al., PNAS 106, 6044 (2009).

### Modal / near-field

- Jackson, Classical Electrodynamics (1999) Ch. 8; Polder & Van Hove, Phys. Rev. B 4, 3303 (1971); Basu et al., Int. J. Energy Res. 33, 1203 (2009).

### Monte Carlo methods

- Howell et al., Thermal Radiation Heat Transfer (5th ed.).

## 💬 Feedback & Issues

Found a bug or have a suggestion? Open an issue at:

https://github.com/Jobeer1/LWIR-thermal-panels/issues

Include the simulation parameters, expected vs. actual output, Python/OS version, and any error logs.

## 📄 License

MIT License — see LICENSE.

Physics model: Anisotropic radiative exchange with thin-film, temperature-dependent optics, roughness BRDF, modal-cutoff, RCWA-diffraction (offline), and near-field corrections. Status: Active development — see DEVELOPMENT_NOTES.md for the consolidated roadmap and change history.